MERLIN CONTROLS CORPORATION

Merlin Network Communications Protocol

Version 1.1

Doug Dale

June 3, 1994

Merlin Network Communications Protocol (MNCP) is intended for use on networks that have one master device and up to 255 slave devices.

1. MNCP Packet Definition

The basic MNCP packet definition is made up of the following parts:

<start><addr><control><data><checksum><end>

Each of these segments is defined as follows:

<start> - Two character start sequence consisting of ASCII DLE and STX.

<addr> - One byte address ranging from 0  to 255.

<control> - One byte of control information.

<data> - 0 to 250 bytes of data.

<checksum> - Two bytes of checksum information.

<end> - Two character end sequence consisting of ASCII DLE and ETX.

1.1. Packet Start and End Sequences

The start and end sequences designate the boundaries of each packet.  To prevent these sequences from appearing within the packet, character stuffing is used.  The sender inserts a second DLE character before each  DLE that appears between the start and end sequences.  The extra DLEs are then removed by the receiver.

1.2. Packet Address

The packet address is a one byte value that indicates a unique slave address.  Return packets from slaves to the master device should also have the slave address in the address field.

A packet sent from the master with a 0 as the packet address is considered a broadcast packet that should be processed by all slaves.  Slaves should not send any response upon receiving a broadcast packet.

1.3. Packet Control

The packet control byte consists of two four bit fields.  The upper four bits contain a sequence number.  The lower four bits indicate the packet type as follows:

0000 - Reset

0001 - Data

0010 - Acknowledge

0011 - Acknowledge with data

0010 to 1111 - Reserved

The use of the sequence number and packet type is explained in 2. Packet Processing.

1.4. Packet Data

The packet can contain 0 to 250 bytes of data (the 250 byte limit is imposed by the type of checksum MNCP uses).  The use of character stuffing allows binary data to be transmitted.

1.5. Packet Checksum

MNCP uses the two byte Fletcher's checksum to check data integrity.  The checksum includes all data bytes and the address and control bytes.  The checksum does not include the DLE characters added during character stuffing.

The checksum is calculated by the following procedure:

sum1 = 0;

sum2 = 0;

for (i = 0; i < length; i++) {

sum1 = (sum1 + message[i]) % 255;

sum2 = (sum2 + sum1) % 255;

}

c1 = 255 - ((sum1 + sum2) % 255;

c2 = 255 - ((sum1 + c1) % 255;

c1 and c2 are then appended to the message.  On the other end, the check is made by adding all of the message bytes and the two checksum bytes modulo 255.  If the result is 0, the data is valid.

2. Packet Processing

The key to understanding the packet processing procedure is understanding the role of the packet sequence number.  This four bit number helps insure that the master and slave are synchronized and that packets are not unnecessarily repeated because of missed or garbled packets.  The Reset-type packet is used to initially synchronize the master and slave.  Keep these thoughts in mind as you examine the next two subsections which describe the process  using a C-like pseudocode.  Slave packet processing will be described first, because it is simpler and a knowledge of the slave's responsibility will aid in understanding the master device's processing procedure.  For the sake of description, think of these processes as separately executing tasks that pass information to and from the main task.

2.1. Slave Packet Processing

slaveaddr = get_slave_address();

sequence = 0;

outpacket = build_ack(slaveaddr, sequence, NULL);

set_timeout(-1);					/* Never timeout.*/

while(True) 					/* Loop forever. */

{

wait_for_packet_from_network(inpacket);  	/* Wait for packet arrival. */

if (checksum_ok(inpacket))

{

parse_packet(inpacket, addr, inseq, intype, data);

if (addr  == slaveaddr)

{

if (intype == Reset)

{

sequence = 0;	/* Valid reset packet. */

outpacket = build_ack(slaveaddr, sequence, NULL);

send_packet_to_network(outpacket);

}

else if (intype == Data)

{

if (inseq != sequence)	/* New data packet. */

{

sequence = inseq;

pass_data_to_main_task(data);

wait_for_data_from_main_task(data);

outpacket = build_ack(slaveaddr, sequence, data);

}

send_packet_to_network(outpacket);

}

}

}

}

2.2. Master Packet Processing

send_reset_packet_to_all_slaves();

sequence = 0;

while (True)					/* Loop forever. */

{

wait_for_data_from_main_task(data, addr, timeout, retries);

sequence = (sequence + 1) % 16;

outpacket = build_data(addr, sequence, data);	/* Build data packet. */

tries = 1+retries;

send_packet_to_network(outpacket);	/* Send out data packet. */

set_timeout(timeout);

done = 0;

while (done == 0)				/* Wait for response or failure. */

{

status = wait_for_packet_from_network(inpacket);

if (status != Timeout)

{				/* Got some response. */

if (!checksum_ok(inpacket))

continue;		/* Ignore bad packets. */

parse_packet(inpacket, inaddr, inseq, intype, data);

if (inaddr == addr && inseq == sequence &&

(intype == Acknowledge || intype == AcknowledgeData))

{			/* If valid response, pass data. */

pass_data_to_main_task(data);

done = 1;

}

else				/* Else return to start of loop. */

continue;

}

else 					/* Timed out. */

{

tries = tries - 1;

if (tries <= 0)		/* If out of tries, failure. */

{

signal_comm_failure_to_main_task();

done = 1;

}

else				/* Else, resend packet. */

{

send_packet_to_network(outpacket);

reset_timeout();

}

}

}

}

2.3 Description of Functions Used in Pseudocode

This section briefly describes the purpose of the functions used in the pseudocode.  Remember that both the pseudocode and these functions simply provide a means for generally describing the steps necessary to use MNCP.  An actual implementation may vary considerably from this simplified illustration.

get_slave_address()

Returns the network address of the unit.

build_ack()

Builds a packet for a response from the slave.

set_timeout()

Sets the timeout clock to a value.  Passing a negative number indicates that the timeout will not be used.  The timeout clock is used by the wait_for_packet_from_network() function.

wait_for_packet_from_network()

Waits for a sequence of characters bracketed by the start and end sequence.  The packet returned does not include the start and end sequence, nor the stuffed DLE characters.  This function does not check the data integrity.  checksum_ok() performs that function.  If the timeout clock expires, the function returns the value 'Timeout'.

checksum_ok()

Verifies data integrity using the checksum.

parse_packet()

Simply separates out the address, sequence, type, and data from the packet.

send_packet_to_network()

Adds the start and end sequence, stuffs DLEs, and sends the packet out on the network.

pass_data_to_main_task()

Passes data from a valid packet to the main task.

wait_for_data_from_main_task()

Waits for the main task to supply data.

send_reset_packet_to_all_slaves()

Sends reset packets to insure the master and all slaves are synchronized.

build_data()

Builds a packet to be sent out from the master.

signal_comm_failure_to_main_task()

Informs the main task that it was not possible to complete the latest communication.

reset_timeout()

Resets the timeout clock to the value passed by set_timeout().