Merlin Controls Corporation

ACL-I/O

Version 1.0

Doug Dale

July 8, 1993

This document describes the general extensions to the Astronomy Command Language (ACL) for general purpose I/O devices.

1. Commands

There are no command extensions for ACL-I/O.

2. Identifiers

The following identifiers represent  ACL-I/O values.  Some values are readable and writeable, while some are read only.  The identifier's read-write type is given at the end of the description as [rw] readable and writeable, [r] read only, or [d] device dependent.

in[]

Description : Input line status. [r]

Type : integer.

Range : 0 or 1.

Index range : 0 to 15.

out[]

Description : Output line status. [rw]

Type : integer.

Range : 0 or 1.

Index range : 0 to 15.

atod[]

Description : Analog to digital converter value. [r]

Type : integer.

Range : Device dependent.

Index range : 0 to 7.

dtoa[]

Description : Digital to analog converter value. [rw]

Type : integer.

Range : Device dependent.

Index range : 0 to 7.

3. Status

There are no predefined status flags for ACL-I/O.  Flags 0 through 15 are reserved for future use.

4. Errors

Error codes 100 through 199 are reserved by ACL-I/O.  Codes 200 and up may be used for device specific errors.