MERLIN CONTROLS CORPORATION

ACL-Dome: Version 1.1

Doug Dale

June 2, 1994

This document describes the general extensions to the Astronomy Command Language (ACL) for dome control devices.

1. Commands

The commands listed below are extensions to the ACL Core.

move east|west|north|south

Starts motion in the specified direction.

stop

Stops all motion initiated by a move or goto command and returns to the rates specified by trackaz and trackalt.

goto [telescopeaz=<value>] [telescopealt=<value>]

Moves dome to the current telescopeaz and telescopealt positions.  Either or both of those values may be supplied with the command.  See 2. Identifiers for more information on telescopeaz and telescopealt.

open

Opens dome doors.

close

Closes dome doors.

home find|park|setup

Supports a dome 'reference point' to initialize encoders.  home find tells the dome controller to search for a home position sensor and initialize the encoder appropriately.  home park sends the dome to the home position.  home setup is used to teach the dome controller the encoder reading that corresponds to the  home sensor position.

2. Identifiers

The following identifiers represent  ACL-Dome values.  Some values are readable and writeable, while some are read only.  The identifier's read-write type is given at the end of the description as [rw] readable and writeable, [r] read only, or [d] device dependent.

az

Description : The current azimuth of the dome opening. [rw]

Type : degrees

Range : 000d00m00s to 359d59m59s

alt

Description : The current altitude of the dome opening. [rw]

Type : degrees

Range : 00d00m00s to 90d00m00s

telescopeaz

Description : The desired azimuth of the dome opening. [rw]

Type : degrees

Range : 000d00m00s to 359d59m59s

telescopealt

Description : The desired altitude of the dome opening. [rw]

Type : degrees

Range : 00d00m00s to 90d00m00s

speed

Description : The rate used for the move command. [rw]

Type : degrees

Units : Degrees/second.

Range : Device specific.

trackaz

Description : Set to a non-zero rate if constant tracking is desired. [rw]

Type : degrees

Units : Degrees/second.

Range : Device specific.

trackalt

Description : Set to a non-zero rate if constant tracking is desired. [rw]

Type : degrees

Units : Degrees/second.

Range : Device specific.

3. Status

Some of the status flags are predefined for ACL-Dome.  Flags 16-31 are left for device specific flags.  Flags 0-15 are predefined or reserved and are described below:

0 - Reserved.

1 - Goto in progress if 1. [r]

2 - Home in progress if 1. [r]

3 - Last home search failed if 1. [r]

5 to 15 - Reserved.

4. Errors

Error codes 100 through 199 are reserved by ACL-Dome.  Codes 200 and up may be used for device specific errors.