Merlin Controls Corporation

Astronomy Command Language

Version 1.1

Doug Dale

July 29, 2026

The Astronomy Command Language (ACL) is intended as a general interface language for use in telescope control systems and related devices.  ACL is designed to provide a basic interface with the flexibility to be extended to meet the varied requirements of different devices.

ACL and its extensions can be best described by a three level hierarchy:

1. ACL core

2. General extensions

3. Device specific extensions

The ACL core is defined in this document.  Basic structure, syntax, commands and identifiers are defined that are to be used in any device that supports ACL.

At the next level are the general extensions.  These are extensions defined by Merlin Controls to cover a generic definition of a device.  ACL-Telescope is one example of such a definition.  If a specific device has a generic extension under which it falls, it should conform to the definitions described in that extension.  For example, a telescope that supports ACL would support the ACL core and the extensions described in ACL-Telescope.

Finally, if a device has features or functions that are not described in the general extensions for that device type, the implementation may define new, device specific extensions with new keywords.  An ACL convention for these device specific keywords is that they begin with an 'x' and an underscore ('x_function' for example).  This prevents any conflicts in the event that the ACL core or the general extensions are amended in the future.

If a device using one of the general extensions does not use one of the keywords defined in that extension, the appropriate error code should be returned (see 5. Errors).

A device that does not have a general extension to describe it can define all of its commands and identifiers with device specific keywords.

1. ACL Basics

ACL commands are made up of simple one-line 'statements' with English-like keywords and identifiers.  Keywords define commands that direct the device to perform some function, or to return some sort of information.  Identifiers represent the various values that need to be communicated between the host and the device.

Devices using ACL give one-line responses that indicate failure or success and return any requested information.  In both commands and responses, spaces provide delimiters between elements and each line is terminated with a newline character.

2. ACL Basic Elements

This section describes the basic elements of ACL.  These elements should be considered reserved, and their meanings should not change for any device that uses ACL.

2.1. Operators

There are only two operators in ACL.  The first is the assignment operator which is represented by an equal sign ('=').  Use of this operator is limited to commands that have a syntax that allows it (see 2.2. Commands).  The second is the indexing operator which is represented by two square brackets ('[' and ']') surrounding an integer.  This operator is used to index array identifiers (see the flag identifier in 2.4. Identifiers as an example).

2.2. Commands

set <id>=<value>

Assigns the <value> to the given <id>.  The type and range of the value is determined by the <id>.

get <id>

Returns the current value of the given <id>.  The type and range of the return value is determined by the <id>.

status

Returns a hexadecimal representation of the 32-bit status word.  See 4. Status for more information.

device

Returns a string that describes the device and software version using the following format:

Company Name<tab>Device Name<tab>1.23

2.3. Device Responses

A device using ACL will respond in one of  the following ways:

ok [<value>]

An ok indicates that a command was successful.  If the command requests information, the information will follow the ok.

error <number>

Errors are indicated by the returned <number>.  See 5. Errors for more information.

2.4. Identifiers

The following identifiers represent the basic ACL values.  Some values are readable and writeable, while some are read only.  The identifier's read-write type is given at the end of the description as [rw] readable and writeable or [r] read only.

flag[]

Description : 32 element flag array.  Some flags are read only while others are read/write depending on the nature of the flag.  See 4. Status for more information.

Type : integer

Range : 0 or 1

Index Range : 0 to 31

baud

Description : The current communications baud rate. [rw]

Type : integer

Range : 9600 to maxbaud

maxbaud

Description : The maximum baud rate the device can achieve. [r]

Type : integer

3. Types

ACL recognizes four basic types:

integer

Base 10 integer value.  Any identifier using this type should specify units as well as range.

real

Real value.  Any identifier using this type should specify units as well as range.

degree

Signed value in degrees.  These values are broken up into three parts, degrees, minutes, and seconds with each part specified by a trailing 'd', 'm', or 's'.  In this format, degrees and minutes are expressed as integers while seconds are expressed as a real value.

Examples:

+45d16m23s

-12d0m09s

Degree values may also be specified as real degrees.

Example:

-34.23456d

hour

Similar to the degree type with the following exceptions:  the value is unsigned and the first part is hours rather than degrees.

Examples:

2h45m34.2s

19h12m59s

9.4532h

string

This type allows arbitrary strings.  Usage for identifiers with this type should be well defined, particularly if the string contains non-ASCII data.

4. Status

ACL defines a 32-bit word to indicate status and allow the manipulation of software flags.  Individual flags can be accessed using the flag identifier.  The status command allows the host  to get the entire 32-bit status word from the device as a hexadecimal value.

5. Errors

Command errors are indicated by an error return string.  The number returned in that string indicates the type of error encountered.  Error numbers 100 and up are left for device specific errors.  Error numbers 0-99 are predefined or reserved, and are described below:

0 - No error.

1 - Undefined error.

2 - Syntax.

3 to 9 - Reserved.

10 - Type mismatch.

11 - Range.

12 - Value is read only.

13 - Unsupported command.

14 - Unsupported identifier.

15 - Command inactive.

16 to 99 - Reserved.