MERLIN CONTROLS CORPORATION

ArchImage Command Language -Version 1.2

Doug Dale

July 12, 1994

Merlin's ArchImage uses ACL-Telescope  (ACL-T) as a basis for its command set.  Please see Merlin's document ACL Telescope - Version 1.1 for details on ACL-T.  This document lists all ArchImage specific commands and identifiers.

1. Commands

x_beep

Produces a beep at the handpaddle.

x_focus [x_focuser=<value>] x_in|x_out|x_off

Starts or stops the focus motor.  If x_focuser is not specified in command the current value of the x_focuser identifier is used (see 2. Identifiers for more information on x_focuser).

x_ieca x_point|x_clear

x_ieca x_point adds a correction point to the current IECA data set, while x_ieca x_clear clears the data set.

x_jog north|south|east|west

x_jog moves the telescope one arcminute in the specified equatorial direction.

x_keystatus

x_keystatus returns a 32 bit hexadecimal word that shows the currently pressed hand paddle keys.  x_key[] is an identifier that allows keys to be accessed individually, see its description in section 2 for more information.

x_pec x_learn|x_clear

x_pec x_clear clears the periodic error correction data.  x_pec x_learn starts a learn sequence.  A learn can only be performed when periodic error correction is enabled (see 3.Status for details).

2. Identifiers

x_ccdguide

Description : Motion rate for CCD guider. [rw]

Type : degrees.

Units : Degrees/second.

Range : 0 to 1 degrees/second.

x_correction_dec

Description : Returns the IECA declination correction. [r]

Type: degrees.

x_correction_ha

Description : Returns the IECA hour angle correction. [r]

Type: degrees.

x_display

Description : Hand paddle display string (4 characters). [rw]

Type : string.

x_focuser

Description : The number of the currently active focuser. [rw]

Type : integer.

Range : 0 or 1.

x_focuspos[]

Description : Position of the focus motor. [rw]

Type : integer.

Range : 0 to 255.

Index range : 0 or 1.

x_focusspeed[]

Description : Speed of the focus motor. [rw]

Type : integer.

Range : 0 to 100.

Index range : 0 or 1.

x_homerange

Description : Search range for the home command. [rw]

Type : degrees.

Range : 5d00m00s to 45d00m00s.

x_homespeed[]

Description : x_homespeed[0] is the speed used for the home sensor search.  x_homespeed[1] if the speed used for determining the home signal center. [rw]

Type : degrees.

Units : Degrees/second.

Range : 0 to 1 degree/second.

x_key[]

Description : x_key allows access to individual hand paddle keys.  Each flag is set when its corresponding key is pressed and cleared when that key is released.  The user can also clear the flags.  The x_keystatus command provides access to the flags in a single 32 bit word.  See section 5 for key assignments. [rw]

Type : integer.

Range : 0 or 1.

Index range : 0 to 31.

x_reticle[]

Description : Reticle brightness. [rw]

Type : integer.

Range : 0 to 100.

Index range : 0 or 1.

x_slewdist[]

Description : Distance remaining in current slew for specified axis.  In alt-az mode, axis 0 corresponds to azimuth, 1 to altitude.  In equatorial mode, axis 0 corresponds to right ascension, 1 to declination. [r]

Type : degrees.

Range : -360d00m00s to 360d00m00s.

Index range : 0 or 1.

3. Status

ArchImage's status word is described below.  Flags 0 through 15 are standard for ACL-T, while 16-31 are specific to ArchImage.  Unused flags are returned as zeros.

0 - Tracking enabled(1)/disabled(0). [rw]

1 - Goto in progress if 1. [r]

2 - Home in progress if 1. [r]

3 - Last home search failed if 1. [r]

4 - Alt-az (0)/equatorial(1) mount. [rw]

5 - 0.

6 - 0.

7 - 0.

8 - 0.

9 - 0.

10 - 0.

11 - 0.

12 - 0.

13 - 0.

14 - 0.

15 - 0.

16 - IECA enabled (1)/disabled (0). [rw]

17 - 0.

18 - 0.

19 - Found home if 1. [r]

20 - Horizon check enabled (1)/disabled (0). [rw]

21 - German mount mode enabled (1)/disabled (0). [rw]

22 - Allow direction keys to pass through from handpaddle if 1 (see x_key[]). [rw]

23 - PEC enable (1)/disabled (0). [rw]

24 - PEC learning (1)/correcting (0) (valid only when bit 23 is 1). [r]

25 - 0.

26 - 0.

27 - 0.

28 - 0.

29 - 0.

30 - 0.

31 - 0.

4. Errors

200 - Model.

201 - Model - full.

210 - Operation cannot be performed in alt/az mode.

211 - Operation cannot be performed in equatorial mode.

5. Hand Paddle Keys

The x_keystatus command and x_key identifier allow access to the status of the hand paddle keys.  The status bits are assigned as follows:

0 - North.

1 - South.

2 - East.

3 - West.

4 - Mode.

5 - Enter.

6 to 31 - Unused, always 0.