Merlin Controls Corporation

ACL-Filter Wheel

Version 1.0

Doug Dale

July 19, 1997

This document describes the general extensions to the Astronomy Command Language (ACL) for filter wheel control devices.

1. Commands

The commands listed below are extensions to the ACL Core.

goto <value>

Moves the wheel to the specified filter.

2. Identifiers

The following identifiers represent  ACL-Filter Wheel values.  Some values are readable and writeable, while some are read only.  The identifier's read-write type is given at the end of the description as [rw] readable and writeable, [r] read only, or [d] device dependent.

filter

Description : The number of the current filter. [r]

Type : integer.

Range : 1 to filtercount.

filtercount

Description : The number of  filters on the wheel. [r]

Type : integer.

filtertype[]

Description : Filter descriptions. [r]

Type : string.

Index Range : 1 to filtercount.

3. Status

Some of the status flags are predefined for ACL-Filter Wheel.  Flags 16-31 are left for device specific flags.  Flags 0-15 are predefined or reserved and are described below:

0 - Reserved.

1 - Goto in progress if 1. [r]

2 to 15 - Reserved.

4. Errors

Error codes 100 through 199 are reserved by ACL-Filter Wheel.  Codes 200 and up may be used for device specific errors.