Merlin Controls Corporation

ACL-Telescope

Version 1.1

Doug Dale

July 29, 2026

This document describes the general extensions to the Astronomy Command Language (ACL) for telescopes.

1. Commands

The commands listed below are extensions to the ACL Core.

move north|south|east|west

Starts motion in the specified direction.

stop north|south|east|west

Stops motion in the specified direction.  If no direction is specified, motion in all axes is stopped.  Any time this command is issued, any goto that is in progress is halted (see the goto command below).

sync

Matches the telescope's right ascension and declination to the current object right ascension and declination.

home find|park|setup

Supports a telescope 'reference point' to initialize encoders.  home find tells the telescope to search for home position sensors and initialize the encoders appropriately.  home park sends the telescope to the home position.  home setup is used to teach the telescope the encoder readings that correspond to the  home sensor positions.

goto [<id>=<value>] [<id>=<value>]

Moves telescope to specific coordinates.  These coordinates may have been specified previously, or they may be specified along with the command.  The following rules apply to the goto command in telescopes (rules may vary in other devices):

-	No parameters - Moves to the current objectra and objectdec.

-	objectra=<value> objectdec=<value> - Moves to the specified equatorial coordinates.  If one of the two is omitted, moves to the specified coordinate in one axis and the current equatorial object coordinate in other axis.

-	objectaz=<value> objectalt=<value> - Moves to the specified alt-az coordinates.   If one of the two is omitted, moves to the specified coordinate in one axis and the current alt-az object coordinate in other axis.  Some telescopes may not support alt-az coordinates.

See 2.4. Identifiers for more information on objectra, objectdec, objectaz, and objectalt.

2. Identifiers

The following identifiers represent  ACL-Telescope values.  Some values are readable and writeable, while some are read only.  The identifier's read-write type is given at the end of the description as [rw] readable and writeable, [r] read only, or [d] device dependent.

ra

Description : The current right ascension of the telescope. [d]

Type : hours

Range : 00h00m00.0s to 23h59m59.9s

dec

Description : The current declination of the telescope. [d]

Type : degrees

Range : -90d00m00s to +90d00m00s

ha

Description : The current hour angle of the telescope. [d]

Type : hours

Range : 00h00m00.0s to 23h59m59.9s

alt

Description : The current altitude of the telescope. [d]

Type : degrees

Range : -90d00m00s to +90d00m00s

az

Description : The current azimuth of the telescope. [d]

Type : degrees

Range : 000d00m00s to 359d59m59s

objectra

Description : Object right ascension. [rw]

Type : hours

Range : 00h00m00.0s to 23h59m59.9s

objectdec

Description : Object declination. [rw]

Type : degrees

Range : -90d00m00s to +90d00m00s

objectalt

Description : Object altitude. [rw]

Type : degrees

Range : -90d00m00s to +90d00m00s

objectaz

Description : Object azimuth. [rw]

Type : degrees

Range : 000d00m00s to 359d59m59s

sidereal

Description : The current sidereal time. [rw]

Type : hours

Range : 00h00m00.0s to 23h59m59.9s

latitude

Description : The latitude. [d]

Type : degrees

Range : -90d00m00s to +90d00m00s

speed

Description : The rate used for the move command. [rw]

Type : degrees

Units : Degrees/second.

Range : Telescope specific.

trackha

Description : Hour angle track rate offset.  This value is summed with the sidereal track rate. [rw]

Type : degrees

Units : Degrees/second.

Range : Telescope specific.

trackdec

Description : Declination track rate. [rw]

Type : degrees

Units : Degrees/second.

Range : Telescope specific.

3. Status

Some of the status flags are predefined for ACL-Telescope.  Flags 16-31 are left for device specific flags.  Flags 0-15 are predefined or reserved and are described below:

0 - Tracking enabled(1)/disabled(0). [rw]

1 - Slew in progress if 1. [r]

2 - Home in progress if 1. [r]

3 - Last home search failed if 1. [r]

4 - Alt-az(0)/equatorial(1) mount.

5 - Reserved.

6 - Reserved.

7 - Reserved.

8 - Passed high az/ra limit if 1. [r]

9 - Passed low az/ra limit if 1. [r]

10 - Passed high alt/dec limit if 1. [r]

11 - Passed low alt/dec limit if 1. [r]

12 - Reserved.

13 - Reserved.

14 - Reserved.

15 - Hit hard limit if 1. [r]

5. Errors

Error codes 100 through 199 are predefined or reserved by ACL-Telescope.  Codes 200 and up may be used for device specific errors.

100 - Goto - illegal parameter(s).

101 - Goto - object below horizon.

102 - Goto - object outside limits.

103 to 199 - Reserved.