{

--	+-----------------------------+

--	|	|

--	|	FFT	|	DELPHI UNIT

--	|	|

--	+-----------------------------+}

# unit fft;

# interface

## {

## -- purpose

## -- Ronchi test application of 2D FFT.

## -- notes

## -- Word 2000 source code:

## -- C:\Allmine\Delphi\Diffract\FFT.doc	2026-07-29

## -- modifications

## --	JD2451307	Jim Burrows	first coding.

## --	1931	"	read Sixtests surface file.

## --	1313	"	back to Y-fixed source.

## --	1319	"	spline fit to mirror.

## --	1329	"	profile plot.

## --	1374	"	-> Word 97.

## --	1417	"	add tab pages, masks.

## --	1441	"	chng choose Ritchey hole pair.

## --	1616	"	chose fixed-source location.

## --	1932	"	read new Sixtests' surface file, fix spline fit.

## --	2467	"	first change in long time.  Two display pages, azimuthal

## --			Zernike polys.

## --	2780	"	zone parms display for all R, b.

## --	3099	"	remove QDialogs unit (CLX).

## }

# uses Windows, Messages, SysUtils, Classes, Graphics, Controls, Forms, Dialogs, ExtCtrls, Buttons, StdCtrls;

## type

## TForm1= class( TForm)

### A11_LABEL:	TLabel;

### A22_LABEL:	TLabel;

### A31_LABEL:	TLabel;

### A33_LABEL:	TLabel;

### A42_LABEL:	TLabel;

### A44_LABEL:	TLabel;

### Alias:	TLabel;

### b_label:	TLabel;

### B_OUT:	TLabel;

### BitBtn1:	TBitBtn;

### COLOUR:	TRadioGroup;

### COMPUTE:	TButton;

### CTRST_LABEL:	TLabel;

### ctrst_out:	TLabel;

### cx_label:	TLabel;

### cx_out:	TLabel;

### cy_out:	TLabel;

### d_label:	TLabel;

### d_out:	TLabel;

### f_label:	TLabel;

### f_OUT:	TLabel;

### file_menu:	TMenuItem;

### ft_size:	TLabel;

### Help1:	TMenuItem;

### I_PAGE:	TTabSheet;

### i_view:	TImage;

### inner:	TLabel;

### Inner_label:	TLabel;

### L_label:	TLabel;

### L_out:	TLabel;

### M_PAGE:	TTabSheet;

### MainMenu1:	TMainMenu;

### MASK_PAGE:	TTabSheet;

### mask_type:	TRadioGroup;

### MIRROR_PAGE:	TTabSheet;

### N_label:	TLabel;

### N_OUT:	TLabel;

### nm_label:	TLabel;

### open_setup:	TMenuItem;

### open_file:	TOpenDialog;

### outer:	TLabel;

### Outer_label:	TLabel;

### PAGES:	TPageControl;

### r_label:	TLabel;

### R_OUT:	TLabel;

### rms_label:	TLabel;

### rms_out:	TLabel;

### ronchi_label:	TLabel;

### ronchi_lines:	TLabel;

### save_setup:	TMenuItem;

### save_file:	TSaveDialog;

### save_IMAGE:	TMenuItem;

### slit_label:	TLabel;

### slit_w:	TLabel;

### source_type:	TRadioGroup;

### STORE:	TButton;

### sw_label:	TLabel;

### sw_out:	TLabel;

### test_PAGE:	TTabSheet;

### test_type:	TRadioGroup;

### toggle:	TButton;

### VIEW:	TImage;

### VIEWS:	TPageControl;

### w_label:	TLabel;

### W_OUT:	TLabel;

### x_label:	TLabel;

### X_OUT:	TLabel;

### y_label:	TLabel;

### Y_OUT:	TLabel;

### ZC_OUT:	TMemo;

### ZC1_LABEL:	TLabel;

### ZC2_LABEL:	TLabel;

### ZC3_LABEL:	TLabel;

### ZL_LABEL:	TLabel;

### ZN_LABEL:	TLabel;

### zone_list:	TListBox;

### zone_n:	TLabel;

### zone_r:	TLabel;

### zr_label:	TLabel;

### procedure B_OUTClick(	Sender: TObject);

### procedure colourClick(	Sender: TObject);

### procedure computeClick(	Sender: TObject);

### procedure cx_OUTClick(	Sender: TObject);

### procedure cy_OUTClick(	Sender: TObject);

### procedure D_OUTClick(	Sender: TObject);

### procedure f_OUTClick(	Sender: TObject);

### procedure FormCreate(	Sender: TObject);

### procedure Help1Click(	Sender: TObject);

### procedure innerClick(	Sender: TObject);

### procedure l_OUTClick(	Sender: TObject);

### procedure mask_typeClick(	Sender: TObject);

### procedure N_OUTClick(	Sender: TObject);

### procedure open_setupClick(	Sender: TObject);

### procedure OUTerClick(	Sender: TObject);

### procedure r_outClick(	Sender: TObject);

### procedure ronchi_linesClick(	Sender: TObject);

### procedure save_imageClick(	Sender: TObject);

### procedure save_setupClick(	Sender: TObject);

### procedure slit_wClick(	Sender: TObject);

### procedure SOURCE_TYPEClick(	Sender: TObject);

### procedure sw_OUTClick(	Sender: TObject);

### procedure test_typeClick(	Sender: TObject);

### procedure toggleClick(	Sender: TObject);

### procedure W_OUTClick(	Sender: TObject);

### procedure X_OUTClick(	Sender: TObject);

### procedure Y_OUTClick(	Sender: TObject);

### procedure zc_outEnter(	Sender: TObject);

### procedure zc_outExit(	Sender: TObject);

### procedure zone_nClick(	Sender: TObject);

### procedure zone_rClick(	Sender: TObject);

### private

### procedure set_zone_data;

## end;

## var

## Form1: TForm1;

# implementation

## {$R *.DFM}

# uses FOURIER5, jwblib, Math, TEXT_IO, zpac2;

## type

### t_source=	( fixed, moving, star);

### t_mask=	( couder, none, pinstick, ritchey);

### t_test=	( ke, ronchi, slit);

### tz_names=	( a11, a31, a22, a42, a33, a44);

### tz_coeff=	array[ tz_names] of complex;

### vect100=	array[ 1.. 100] of extended;

## const

### two_pi=	2.0* pi;

### wave_table:	array[ 0.. 5] of extended=(

### 632.8e-6, 570.0e-6, 500.0E-6, 495.0e-6, 490.0e-6, 550.0e-6);

## var

BIT_MAP:	TBitmap;

### bp1:	extended;

### coef:	vect100;

### data_read:	boolean= false;

### dcap_y:	extended;

### deform_actual:	extended;

### deform_save:	extended;

### DEFORMATION:	EXTENDED= -1.0;

### diam:	EXTENDED= 200.0;

### DIAM_actual:	EXTENDED;

### DIAM_save:	EXTENDED;

### f:	EXTENDED= 2400.0;

### get_data:	boolean= true;

### have_help:	boolean= false;

### hlp_file_name:	pchar;

### item_index:	integer;

### knots:	vect100;

### lines_mm:	EXTENDED;

### mask:	t_mask;

### mask_height:	EXTENDED;

### mirror:	text;

### n_knots:	integer;

### N_POINTS:	INTEGER= 256;

### order:	integer;

### pin_space:	EXTENDED;

### rms_factors:	array[ tz_names] of extended=

#### ( 4.0, 8.0, 6.0, 10.0, 8.0, 10.0);

### ROC:	EXTENDED= 2400.0;

### ROC_actual:	EXTENDED;

### ROC_save:	EXTENDED;

### series:	C2_ARRAY;

### setup_file:	text;

### slit_width:	extended;

### source:	t_source= fixed;

### source_width:	EXTENDED= 0.05;

### sqr_roc:	extended;

### t_width:	EXTENDED= 220.0;

### test:	t_test;

### transform:	C2_ARRAY;

### use_data:	boolean= false;

### WAVELENGTH:	extended;

### XF:	EXTENDED= 2402.043;

### Xs:	EXTENDED= 2400.0;

### y_mm:	EXTENDED= 0.0;

### z_coeff:	tz_coeff=(

#### ( x: 0.0; y: 0.0),

#### ( x: 0.0; y: 0.0),

#### ( x: 0.0; y: 0.0),

#### ( x: 0.0; y: 0.0),

#### ( x: 0.0; y: 0.0),

#### ( x: 0.0; y: 0.0));

### z_strings:	array[ tz_names] of string=(

#### 'a11', 'a31', 'a22', 'a42', 'a33', 'a44');

### zone:	integer;

### zone_space:	EXTENDED;

### zone_y:	EXTENDED;

### zones:	integer;

# {-- For Word - start of code.}

## function height_at( y, z: extended):  extended;

### var

#### a:	extended;

#### dbiaty:	vect100;

#### i:	integer;

#### izn:	tz_names;

#### j:	integer;

#### left:	integer;

#### p:	extended;

#### r:	extended;

#### r2:	extended;

#### rho:	extended;

#### rho2:	extended;

#### x:	extended;

#### z_p:	array[ tz_names] of complex;

#### z1, z2:	complex;

## begin

### r2:= sqr( y)+ sqr( z);

### if use_data then begin

#### {-- Interpolation hassle.  Try splines.}

#### p:= sqrt( r2);

#### interv( knots, n_knots, p, left, i);

#### bsplvd( knots, order, p, left, dbiaty, 1);

#### x:= 0.0;

#### for i:= 1 to order do begin

##### j:= left- order+ i;

##### if j> 0 then x:= x+ dbiaty[ i]* coef[ j];

#### end;

### end else x:= r2/( roc+ sqrt( sqr_roc- bp1* r2));

### {-- Working azimuthal Zernike coefficients. 2362:  use complex numbers, dummy!  2479:  set up Z_P as conjugates to help the final dot product.  Don't forget c(xy)= c(x)c(y), [c() = conjugate].}

### R:= sqrt( r2);

### RHO:= 2.0* R/ diam;

### RHO2:= SQR( RHO);

### z1:= cmplx( y/ r, -z/ r);

### z2:= z1;

### z_p[ a11]:= c_scale( rho, z2); 	{-- }

### z_p[ a31]:= c_scale( rho*( 3.0* rho2- 2.0), z2); 	{-- 33-2}

### z2:= mul( z1, z1);

### z_p[ a22]:= c_scale( rho2, z2); 	{-- 2}

### z_p[ a42]:= c_scale( rho2*( 4.0* rho2- 3.0), z2); 	{-- 44-32}

### z2:= mul( z1, z2);

### z_p[ a33]:= c_scale( rho* rho2, z2); 	{-- 3}

### z2:= mul( z1, z2);

### z_p[ a44]:= c_scale( sqr( rho2), z2); 	{-- 4}

### a:= 0.0;

### for izn:= a11 to a44 do a:= a+ re( mul( z_coeff[ izn], z_p[ izn]));

### result:= x+ a;

## end; {-- height_at}

## function phase( y, z: extended): extended;

### var

#### a, b, c, d:	extended;

#### x:	extended;

## begin

### x:= height_at( y, z);

### a:= sqr( x)+ sqr( y)+ sqr( z);

### b:= 2.0* roc* x;

### c:= 2.0* xf* x;

### d:= 2.0* y_mm* y;

### case source of

#### fixed: begin

##### {-- Phase difference is source (R, 0, 0) -> point (x, y, z) -> (X, 0, 0) vs. R + X.}

##### b:= a- b;

##### c:= a- c;

##### a:= b/( roc+ sqrt( sqr_roc+ b))+ c/( xf+ sqrt( sqr( xf)+ c));

#### end;

#### moving: begin

##### {-- Phase difference is source (X, Y, 0) -> point (x, y, z) -> (X, -Y, 0) vs. 2*sqrt(X²+Y²).}

##### b:= a- c- d;

##### c:= a- c+ d;

##### d:= sqr( xf)+ sqr( y_mm);

##### a:= sqrt( d);

##### a:= b/( sqrt( b+ d)+ a)+ c/( sqrt( c+ d)+ a);

#### end;

#### star: begin

##### {-- Phase difference is (0, y, z) -> mirror point (x, y, z) -> (X, 0, 0) vs. X.}

##### b:= a- c;

##### a:= b/( sqrt( b+ sqr( xf))+ xf)- x;

#### end;

### end;

### phase:= two_pi* frac( a/ wavelength);

## end; {-- phase}

## function rms:  extended;

### {-- Returns RMS (nm) for current values of D, R, b, Zernike coefficients.}

### var

#### izn:	tz_names;

#### ms:	extended;

## begin

### {-- RMS from b.}

### ms:= IntPower( diam, 4)*( deformation+ 1.0)/ 768.0/ IntPower( roc, 3);

### ms:= sqr( ms)/ 5.0;

### {-- RMS from Zernike coefficients.}

### for izn:= a11 to a44 do ms:= ms+ sqr( c_abs( z_coeff[ izn]))/ rms_factors[ izn];

### result:= 1.0e6* sqrt( ms);

## end; {-- rms}

## procedure set_pin_space;

### var

#### a, b:  extended;

## begin

### a:= ln( 0.5* t_width)/ ln( 10.0);

### b:= frac( 1.0+ frac( a));

### {-- 1 (.1) 1.2 (.2) 2.5 (.5) 5 (1) 10}

### if b< 0.08 then pin_space:= 0.1 else if b< 0.4 then pin_space:= 0.2 else if b< 0.7 then pin_space:= 0.5 else pin_space:= 1.0;

### pin_space:= pin_space* exp( int( a- b)* ln( 10.0));

## end; {-- set_pin_space}

## function zc_str( a:  extended; i:  integer):  string;

### var

#### s:	string;

## begin

### s:= FormatFloat( '0.0', a);

### while length( s)< i do s:= ' '+ s;

### result:= s;

## end; {-- zc_str}

## procedure TForm1. b_outClick( Sender: TObject);

## begin

### input_string:= FormatFloat( '0.000', deformation);

### if InputQuery( 'Deformation constant', '', input_string) then begin

#### get_real( deformation);

#### b_out. Caption:= FormatFloat( '0.000', deformation);

#### rms_out. Caption:= FormatFloat( '0.0', rms);

#### set_zone_data;

### end;

## end; {-- b_outClick}

## procedure TForm1. colourClick( Sender: TObject);

## begin

### with colour do begin

#### if( sender= open_setup)or( sender= source_type) then ItemIndex:= item_index;

#### wavelength:= wave_table[  ItemIndex];

#### case  ItemIndex of

##### 0:  Color:= clRed;

##### 1:  Color:= clYellow;

##### 2:  Color:= clLime;

##### 3:  Color:= clAqua;

##### 4:  Color:= clBlue;

##### 5:  Color:= clWhite;

#### end;

### end;

### Compute. SetFocus;

## end; {-- colourClick}

## procedure TForm1. computeClick( Sender: TObject);

### var

#### a:	extended;

#### b:	extended;

#### bias_pix:	extended;

#### d:	extended;

#### cap_y:	extended;

#### dy:	extended;

#### i, j, k, l:	integer;

#### i1, i2, j1, j2:	integer;

#### idft_max:	extended;

#### idft_min:	extended;

#### lighted:	longint;

#### n_1:	integer;

#### n_2:	integer;

#### odd_test:	extended;

#### outside:	boolean;

#### r:	extended;

#### r2:	extended;

#### scale_pix:	extended;

#### sum_l:	extended;

#### sum_r:	extended;

#### x_bias:	extended;

#### u1:	extended;

#### u2:	extended;

#### w:	extended;

#### y:	extended;

#### y2:	extended;

#### z:	extended;

#### za:	array[ 1.. 4] of complex;

#### zb:	array[ 1.. 4] of complex;

#### zone_inner:	extended;

#### zone_outer:	extended;

### function cross( u1, u2, v1, v2:  extended):  extended;

### begin

#### cross:= u1* v2- v1* u2;

### end;

### procedure sum_sides;

### begin

#### if y< 0.0 then sum_l:= sum_l+ k else sum_r:= sum_r+ k;

### end;

### function gray( k:  integer):  TColor;

#### var

##### i:	integer;

### begin

#### if k> 255 then i:= 255 else i:= k;

#### case colour. ItemIndex of

##### 0: gray:= Tcolor( i);	{-- Red}

##### 1: gray:= Tcolor( 257* i);	{-- Yellow}

##### 2: gray:= Tcolor( 256* i);	{-- Green}

##### 3: gray:= Tcolor( 65792* i);	{-- Blue-Green}

##### 4: gray:= Tcolor( 65536* i);	{-- Blue}

##### 5: gray:= Tcolor( 65793* i);	{-- White}

#### end;

### end;

### procedure load_24;

#### {-- Transfer quadrant 2, 4 phases.}

### begin

#### zb[ 2]:= za[ 2];

#### zb[ 4]:= za[ 4];

### end;

### procedure load_13;

#### {-- Transfer quadrant 1, 3 phases.}

### begin

#### zb[ 1]:= za[ 1];

#### zb[ 3]:= za[ 3];

### end;

## begin

### ft_size. Caption:= '';

### compute. Caption:= 'Wait - 4';

### Alias. Visible:= False;

### {-- Constants.}

### bp1:= deformation+ 1.0;

### dcap_y:= xf* wavelength/ t_width;

### n_1:= n_points- 1;

### dy:= t_width/ n_1;

### n_2:= n_points div 2;

### r2:= 0.25* sqr( diam);

### sqr_roc:= sqr( roc);

### x_bias:= 0.5* t_width;

### ft_size. Left:= n_2- 50;

### ft_size. Caption:= FormatFloat( '0.000', n_points* dcap_y)+ 
' mm';

### {-- Set up the transform kernel W.}

### if not setup_w( n_points) then begin

#### ShowMessage(

'N has to be a power of 2, make it right and click '+

'Run again.');

#### compute. Caption:= 'Run';

#### exit;

### end;

### {-- Generate transmission function.  I1, I2 are pixel # in y-direction (inc to right), J1, J2 in z-direction (inc down).  (y,z), both positive, is in quadrant IV, and both small implies in Couder zone 1.}

### lighted:= 0;

### case mask of

#### couder: begin

##### zone_inner:= sqr( zone_space);

##### r:= zone_space;

##### if mask_height< r then u1:= sqrt( zone_inner- sqr( mask_height))+ r;

#### end;

#### ritchey: begin

##### a:= 0.5* zone_space;

##### zone_outer:= sqr( zone_y+ a);

##### zone_inner:= sqr( zone_y- a);

#### end;

### end;

### for I:= 0 to n_2- 1 do begin

#### y:= dy*( i+ 0.5);

#### y2:= sqr( y);

#### i1:= i+ n_2;

#### i2:= n_2-i- 1;

#### for j:= 0 to n_2- 1 do begin

##### z:= dy*( j+ 0.5);

##### j1:= j+ n_2;

##### j2:= n_2- j- 1;

##### b:= phase( y, -z); za[ 1]:= cmplx( cos( b), sin( b));

##### b:= phase( -y, -z); za[ 2]:= cmplx( cos( b), sin( b));

##### b:= phase( -y, z); za[ 3]:= cmplx( cos( b), sin( b));

##### b:= phase( y, z); za[ 4]:= cmplx( cos( b), sin( b));

##### for k:= 1 to 4 do zb[ k]:= zero;

##### case mask of

###### couder: begin

a:= y2+ sqr( z);

if( z< mask_heighT)and( a<= r2) then begin

inc( lighted);

k:= trunc( a/ zone_inner);

if k mod 2= 0 then load_24 else load_13;

if k= 0 then begin

if mask_height< r then begin

if cross( y+ r, z, u1, mask_height)< 0.0 then begin

zb[ 2]:= zero; zb[ 4]:= zero;

end else if cross( y- r, z, -u1, mask_height)< 0.0 then

load_13;

end else if( sqr( y+ z)< zone_inner) then load_13;

end;

end;

###### end;

###### ritchey: begin

r2:= sqr( y)+ sqr( z);

if( z< mask_heighT)and( zone_inner< r2)and( r2< zone_outer) then begin

inc( lighted, 2);

load_13; load_24;

end;

###### end;

###### else begin

outside:= y2+ sqr( z)> r2;

if outside then begin

zb[ 2]:= zero; zb[ 4]:= zero;

end else begin

inc( lighted, 2);

load_13; load_24;

end;

###### end;

##### end;

##### series[ i1, j1]:= zb[ 4];

##### series[ i1, j2]:= zb[ 1];

##### series[ i2, j1]:= zb[ 3];

##### series[ i2, j2]:= zb[ 2];

#### end;

### end;

### {-- Should I generate pinstick lines?}

### if mask= pinstick then begin

#### y:= 0.0;

#### k:= round( 5.0/ dy- 0.5); {-- Pin's half length = 5 mm.}

#### repeat

##### i:= round( y/ dy- 0.5);

##### i1:= i+ n_2;

##### i2:= n_2- i- 1;

##### for j:= 0 to k do begin

###### j1:= j+ n_2;

###### j2:= n_2- j- 1;

###### dec( lighted, 2);

###### series[ i1, j1]:= zero;

###### series[ i1, j2]:= zero;

###### series[ i2, j1]:= zero;

###### series[ i2, j2]:= zero;

##### end;

##### y:= Y+ pin_space;

#### until y> 0.5* T_width;

### end;

### lighted:= 2* lighted;

### {-- Compute DFT.}

### compute. Caption:= 'Wait - 3';

### harm2( series, n_points, true, transform);

### {-- Obscure part of image.  First, check for aliasing by summing up total light in 1/32 of transform at edge, if > .3%, probably aliasing.}

### a:= 0.0;

### k:= n_2+ n_2 div 16- 1;

### for i:= n_2 to k do for j:= 0 to n_1 do a:= a+ sqr( c_abs( transform[ i, j]));

### b:= n_points;

### b:= lighted* sqr( b);

### if a> 0.003* b then Alias. Visible:= True;

### if source_width<> 0.0 then a:= 2.0/ source_width;

### for i:= 0 to n_1 do begin

#### cap_y:= dcap_Y*(( i+ n_2)mod N_points- n_2);

#### if source= moving then cap_y:= cap_y- y_mm;

#### {-- D is distance to nearest edge, <0 => dark side.}

#### case test of

##### {-- Knife edge to the right.}

##### ke: d:= y_mm- cap_y;

##### {-- Ronchi grid, black centre if parameter< 0.}

##### ronchi: begin

###### D:= FRAC( 1.0+ FRAC(( CAP_Y- y_mm)* lines_mm));

###### D:= ( ABS( D- 0.5)- 0.25)/ lines_mm;

##### end;

##### {-- Slit (or bar if parameter < 0.}

##### SLIT: d:= slit_width*( 0.5- abs(( cap_y- y_mm)/ slit_width));

#### end;

#### if source_width<> 0.0 then begin

##### d:= a* d;

##### if d< -1.0 then for j:= 0 to n_1 do transform[ i, j]:= zero else if d< 1.0 then begin

###### d:= 0.5*( 1.0+ sin( 0.5* pi* d));

###### for j:= 0 to n_1 do with transform[ i, j] do begin

x:= d* x; y:= d* y;

###### end;

##### end;

#### end else if d< 0.0 then for j:= 0 to n_1 do transform[ i, j]:= zero;

### end;

### {-- 1382:  Nils request:  plot transform.}

### i_view. Picture. Graphic:= bit_map;

### {-- Find maximum.}

### idft_max:= 0.0; idft_min:= c_abs( transform[ 0, 0]);

### for i:= 0 to n_1 do for j:= 0 to n_1 do begin

#### a:= c_abs( transform[ i, j]);

#### if a> idft_max then idft_max:= a;

#### if a< idft_min then idft_min:= a;

### end;

### if idft_max= 0.0 then begin

#### a:= 0.0; b:= 0.0;

### end else if idft_max= idft_min then begin

#### a:= 256.0; b:= 0.0;

### end else begin

#### idft_max:= sqr( idft_max); idft_min:= sqr( idft_min);

#### b:= 256.0/( idft_max- idft_min);

#### a:= -b* idft_min;

### end;

### {-- Plot.}

### compute. Caption:= 'Wait - 2.5';

### with i_view. Canvas do for i:= 0 to N_1 do begin

#### i1:= ( i+ n_2)mod N_points;

#### for j:= 0 to n_1 do begin

##### j1:= ( j+ n_2)mod N_points;

##### k:= trunc( a+ b* sqr( c_abs( transform[ i, j])));

##### Pixels[ i1, j1]:= gray( k);

#### end;

### end;

### {-- Compute inverse transform.}

### compute. Caption:= 'Wait - 2';

### harm2( transform, n_points, false, series);

### {-- Find maximum.}

### idft_max:= 0.0; idft_min:= c_abs( series[ 0, 0]);

### for i:= 0 to n_1 do for j:= 0 to n_1 do begin

#### a:= c_abs( series[ i, j]);

#### if a> idft_max then idft_max:= a;

#### if a< idft_min then idft_min:= a;

### end;

### {-- Bias and scale for pixel output.}

### if idft_max= 0.0 then begin

#### bias_pix:= 0.0; scale_pix:= 0.0;

### end else if idft_max= idft_min then begin

#### bias_pix:= 256.0; scale_pix:= 0.0;

### end else begin

#### idft_max:= sqr( idft_max); idft_min:= sqr( idft_min);

#### scale_pix:= 256.0/( idft_max- idft_min);

#### bias_pix:= -scale_pix* idft_min;

### end;

### {-- Plot.}

### compute. Caption:= 'Wait - 1';

### case mask of

#### couder, ritchey: begin

##### ctrst_out. Visible:= true;

##### CTRST_LABEL. Visible:= true;

##### sum_l:= 0.0; sum_r:= 0.0;

##### if mask= couder then begin

###### odd_test:=( zone mod 2)- 0.5;

###### if zone= 1 then begin

if mask_height< r then begin

{-- (u1, u2) is unit vector along centerline, W is half width.}

b:= sqr( mask_height);

a:= sqrt( zone_inner- b)+ r;

b:= sqrt( sqr( a)+ b);

u1:= a/ b;

u2:= mask_height/ b;

w:= cross( r, 0.0, u1, u2);

end else begin

u1:= 1.0/ sqrt( 2.0);

u2:= u1;

w:= u1* r;

end;

###### end;

##### end;

#### end;

### else

#### ctrst_out. Visible:= false;

#### CTRST_LABEL. Visible:= false;

### end;

### view. Picture. Graphic:= bit_map;

### with view. Canvas do for i:= 0 to N_1 do for j:= 0 to n_1 do begin

#### k:= trunc( bias_pix+ scale_pix* sqr( c_abs( series[ i, j])));

#### Pixels[ i, j]:= gray( k);

#### {-- Contrast computation.}

#### if( mask= ritchey)or( mask= couder) then begin

##### z:= dy*( j- n_2+ 0.5);

##### if abs( z)< mask_height then begin

###### y:= dy*( i- n_2+ 0.5);

###### r2:= sqr( y)+ sqr( z);

###### if mask= ritchey then begin

if( zone_inner< r2)and( r2< zone_outer) then sum_sides;

###### end else begin

{-- Couder mask.}

if zone= 1 then begin

if( r2< zone_inner)and( abs( cross( y, z, u1, u2))< w) then sum_sides;

end else if( trunc( r2/ zone_inner)= zone- 1)and( odd_test* y* z> 0.0) then sum_sides;

###### end;

##### end;

#### end;

### end;

### if ctrst_out. Visible and( sum_l+ sum_r<> 0.0) then ctrst_out. Caption:= FormatFloat( '0.0', 100.0*( sum_l- sum_r)/( sum_l+ sum_r))+ '%' else ctrst_out. Caption:= '';

### compute. Caption:= 'Run';

## end; {-- computeClick}

## procedure TForm1. cx_outClick( Sender: TObject);

## begin

### x_out. Caption:= cx_out. Caption;

### input_string:= cx_out. Caption;

### get_real( xf);

### if source= star then xf:= xf+ 0.5* roc else xf:= xf+ roc;

### set_zone_data;

## end; {-- cx_outClick}

## procedure TForm1. cy_outClick( Sender: TObject);

## begin

### y_out. Caption:= cy_out. Caption;

### input_string:= cy_out. Caption;

### get_real( y_mm);

## end; {-- cy_outClick}

## procedure TForm1. d_outClick( Sender: TObject);

## begin

### input_string:= FormatFloat( '0.0', diam);

### if InputQuery( 'Mirror diameter', 'mm', input_string) then begin

#### get_real( diam);

#### d_out. Caption:= FormatFloat( '0.0', diam);

#### rms_out. Caption:= FormatFloat( '0.0', rms);

### end;

## end; {-- d_outClick}

## procedure TForm1. f_outClick( Sender: TObject);

## begin

### x_out. Caption:= f_out. Caption;

### input_string:= f_out. Caption;

### get_real( xf);

### if source= star then xf:= xf+ 0.5* roc else xf:= xf+ roc;

### set_zone_data;

## end; {-- f_outClick}

## procedure TForm1. FormCreate( Sender: TObject);

### var

#### izn:	tz_names;

#### s1:	string;

#### s2:	string;

## begin

### ForceCurrentDirectory:= true;

### wavelength:= wave_table[ colour. ItemIndex];

### SETLENGTH( SERIES, 512, 512);

### SETLENGTH( TRANSFORM, 512, 512);

### BIT_MAP:= TBitmap. Create;

### BIT_MAP. Width:= n_points; BIT_MAP. Height:= n_points;

### VIEW. Picture. Graphic:= BIT_MAP;

### i_view. Picture. Graphic:= BIT_MAP;

### mask:= pinstick; set_pin_space; zone_y:= 70.0; set_zone_data; test:= ke;

### B_OUT. Caption:= FormatFloat( '0.000', DEFORMATION);

### d_OUT. Caption:= FormatFloat( '0.0', diam);

### Y_OUT. Caption:= FormatFloat( '0.000', y_mm);

### n_OUT. Caption:= IntToStr( n_points);

### R_OUT. Caption:= FormatFloat( '0.000', roc);

### sw_out. Caption:= FormatFloat( '0.000', source_width);

### W_OUT. Caption:= FormatFloat( '0', t_width);

### X_OUT. Caption:= FormatFloat( '0.000', xf- roc);

### for izn:= a11 to a44 do with c_scale( 1.0e6, z_coeff[ izn]) do begin

#### s1:= zc_str( x, 6);

#### s2:= zc_str( y, 7);

#### zc_out. Lines[ ord( izn)]:= s1+s2;

### end;

### rms_out. Caption:= FormatFloat( '0.0', rms);

## end; {-- FormCreate}

## procedure TForm1. Help1Click( Sender: TObject);

### var

#### i:	integer;

## begin

### if not have_help then with open_file do begin

#### FileName:= 'diffract.hlp';

#### Filter:= 'Help files (*.hlp)|*.hlp';

#### Title:= 'Open help file.';

#### if Execute then begin

##### hlp_file_name:= PChar( FileName);

##### have_help:= true;

#### end;

### end;

### i:= 0;

### WinHelp( Form1. Handle, hlp_file_name, help_finder, i);

## end; {-- help1Click}

## procedure TForm1. innerClick( Sender: TObject);

### var

#### A:	EXTENDED;

## begin

### if MASK= RITCHEY then begin

#### A:= ZONE_Y- 0.5* ZONE_SPACE;

#### input_string:= FormatFloat( '0.000', A);

#### if InputQuery( 'Inner zone radius', 'mm', input_string) then begin

##### get_real( a);

##### zone_space:= zone_y- a+ 0.5* zone_space;

##### zone_y:= a+ 0.5* zone_space;

##### inner. Caption:= FormatFloat( '0.00', a);

##### set_zone_data;

#### end;

### end;

## end; {-- innerClick}

## procedure TForm1. l_outClick( Sender: TObject);

## begin

### y_out. Caption:= l_out. Caption;

### input_string:= l_out. Caption;

### get_real( y_mm);

## end; {-- l_outClick}

## procedure TForm1. mask_typeClick( Sender: TObject);

### var

#### i:	integer;

#### si:	string;

## begin

### with mask_type do begin

#### if sender= open_setup then ItemIndex:= item_index;

#### case ItemIndex of

##### 0: begin

###### mask:= couder;

###### if get_data then begin

input_string:= InputBox(

'Couder mask', 'Number of zones', IntToStr( 4));

get_integer( zones);

###### end;

###### zone_space:= 0.5* diam/ sqrt( zones);

###### zone_list. Clear;

###### for i:= 1 to zones do begin

str( i: 2, si);

zone_list. Items[ i- 1]:= si+ ':  '+ FormatFloat( '0.0', zone_space* sqrt( i));

###### end;

###### mask_height:= exp( 0.25* ln( 0.25* wavelength* sqr( roc)* roc));

###### if get_data then begin

zone:= 1;

zone_n. Caption:= '1';

zone_y:= 0.5* zone_space;

###### end;

###### ZN_LABEL. Visible:= true; zone_n. Visible:= true;

###### Inner_label. Visible:= false; inner. Visible:= false;

###### Outer_label. Visible:= false; outer. Visible:= false;

###### ZL_LABEL. Visible:= true; zone_list. Visible:= true;

###### set_zone_data;

##### end;

##### 1: begin

###### mask:= none;

###### ZN_LABEL. Visible:= false; zone_n. Visible:= false;

###### Inner_label. Visible:= false; inner. Visible:= false;

###### Outer_label. Visible:= false; outer. Visible:= false;

###### ZL_LABEL. Visible:= false; zone_list. Visible:= false;

##### end;

##### 2: begin

###### mask:= pinstick;

###### ZN_LABEL. Visible:= false; zone_n. Visible:= false;

###### Inner_label. Visible:= false; inner. Visible:= false;

###### Outer_label. Visible:= false; outer. Visible:= false;

###### ZL_LABEL. Visible:= false; zone_list. Visible:= false;

###### set_pin_space;

##### end;

##### 3: begin

###### mask:= ritchey;

###### if get_data then begin

input_string:= InputBox(

'Ritchey mask with paired holes', 'Hole half-height, mm', FormatFloat( '0.0', 10.0));

get_real( mask_height);

zone_space:= 0.125* diam;

outer. Caption:= FormatFloat( '0.00', zone_space);

inner. Caption:= FormatFloat( '0.0', 0.0);

zone_y:= 0.5* zone_space;

###### end;

###### ZN_LABEL. Visible:= false; zone_n. Visible:= false;

###### Inner_label. Visible:= true; inner. Visible:= true;

###### Outer_label. Visible:= true; outer. Visible:= true;

###### ZL_LABEL. Visible:= false; zone_list. Visible:= false;

###### set_zone_data;

##### end;

#### end;

### end;

### compute. SetFocus;

## end; {-- mask_typeClick}

## procedure TForm1. n_outClick( Sender: TObject);

## begin

### input_string:= IntToStr( n_points);

### if InputQuery( 'Number of points', 'Power of 2 <= 512', input_string) then begin

#### get_integer( n_points);

#### n_out. Caption:= IntToStr( n_points);

#### bit_map. Height:= n_points; bit_map. Width:= n_points;

#### view. Picture. Graphic:= bit_map;

#### i_view. Picture. Graphic:= bit_map;

### end;

## end; {-- n_outClick}

## procedure TForm1. open_setupClick( Sender: TObject);

### var

#### a, b:	extended;

#### ch:	char;

#### izn:	tz_names;

#### s1, s2:	string;

## begin

### with open_file do begin

#### Title:= 'Open setup file.';

#### Filter:=

#### 'Text files (*.set, *.txt)|*.set;*.txt|All files (*.*)|*.*';

#### if Execute then begin

##### AssignFile( setup_file, FileName);

##### reset( setup_file);

##### {-- Comment line, check to see if this is the file.}

##### readln( setup_file, s1);

##### if MessageDlg(

##### 'First line: "'+ S1+ '" - right file?', mtCustom, [mbYes, mbNo], 0)= mrNo then exit;

##### {-- D.}

##### readln( setup_file, s1);

##### input_string:= s1;

##### try

###### get_real( diam);

##### except

###### ShowMessage(

###### 'Expected real number in '+ S1);

###### CloseFile( setup_file);

###### Exit;

##### end;

##### d_out. Caption:= FormatFloat( '0.0', diam);

##### {-- R.}

##### readln( setup_file, s1);

##### input_string:= s1;

##### try

###### get_real( roc);

##### except

###### ShowMessage(

###### 'Expected real number in '+ S1);

###### CloseFile( setup_file);

###### Exit;

##### end;

##### r_out. Caption:= FormatFloat( '0.000', roc);

##### {-- b.}

##### readln( setup_file, s1);

##### input_string:= s1;

##### try

###### get_real( deformation);

##### except

###### ShowMessage(

###### 'Expected real number in '+ S1);

###### CloseFile( setup_file);

###### Exit;

##### end;

##### b_out. Caption:= FormatFloat( '0.000', deformation);

##### readln( setup_file); {-- Zernike heading line.}

##### {-- Zernike coefficients.}

##### for izn:= a11 to a44 do begin

###### readln( setup_file, s1);

###### input_string:= s1;

###### try

get_real( a); get_real( b);

###### except

ShowMessage(

'Expected two real numbers in '+ S1);

CloseFile( setup_file);

Exit;

###### end;

###### z_coeff[ izn]:= c_scale( 1.0E-6, cmplx( a, b));

###### s1:= zc_str( a, 6);

###### s2:= zc_str( b, 7);

###### zc_out. Lines[ ord( izn)]:= s1+ s2;

##### end;

##### {-- n_points.}

##### readln( setup_file, s1);

##### input_string:= s1;

##### try

###### get_integer( n_points);

##### except

###### ShowMessage(

###### 'Expected integer in '+ S1);

###### CloseFile( setup_file);

###### Exit;

##### end;

##### n_out. Caption:= IntToStr( n_points);

##### bit_map. Height:= n_points; bit_map. Width:= n_points;

##### view. Picture. Graphic:= bit_map;

##### i_view. Picture. Graphic:= bit_map;

##### {-- t_width.}

##### readln( setup_file, s1);

##### input_string:= s1;

##### try

###### get_real( t_width);

##### except

###### ShowMessage(

###### 'Expected real number in '+ S1);

###### CloseFile( setup_file);

###### Exit;

##### end;

##### w_out. Caption:= FormatFloat( '0.0', t_width);

##### {-- source_width.}

##### readln( setup_file, s1);

##### input_string:= s1;

##### try

###### get_real( source_width);

##### except

###### ShowMessage(

###### 'Expected real number in '+ S1);

###### CloseFile( setup_file);

###### Exit;

##### end;

##### sw_out. Caption:= FormatFloat( '0.000', source_width);

##### {-- Test type.}

##### readln( setup_file, s1);

##### ch:= upcase( s1[ 1]);

##### case ch of

###### 'k': Item_Index:= 0;

###### 'r': begin

Item_index:= 1;

readln( setup_file, s1);

input_string:= s1;

try

get_real( lines_mm);

except

ShowMessage(

'Expected real number in '+ S1);

CloseFile( setup_file);

Exit;

end;

ronchi_lines. Caption:= FormatFloat( '0.000', lines_mm);

###### end;

###### 's': begin

Item_index:= 2;

readln( setup_file, s1);

input_string:= s1;

try

get_real( slit_width);

except

ShowMessage(

'Expected real number in '+ S1);

CloseFile( setup_file);

Exit;

end;

slit_w. Caption:= FormatFloat( '0.000', slit_width);

###### end;

##### else

###### ShowMessage(

###### 'Expected test type in '+ S1);

###### CloseFile( setup_file);

###### Exit;

##### end;

##### get_data:= false;

##### test_typeClick( open_setup);

##### get_data:= true;

##### {-- Source type.}

##### readln( setup_file, s1);

##### ch:= upcase( s1[ 1]);

##### with source_type do case ch of

###### 'f': ItemIndex:= 0;

###### 'm': ItemIndex:= 1;

##### else

###### ShowMessage(

###### 'Expected source type in '+ S1);

###### CloseFile( setup_file);

###### Exit;

##### end;

##### {-- Source color.}

##### readln( setup_file, s1);

##### ch:= upcase( s1[ 1]);

##### with colour do case ch of

###### 'r': ItemIndex:= 0;

###### 'y': ItemIndex:= 1;

###### 'g': ItemIndex:= 2;

###### 'c': ItemIndex:= 3;

###### 'b': ItemIndex:= 4;

###### 'w': ItemIndex:= 5;

##### else

###### ShowMessage(

###### 'Expected source color in '+ S1);

###### CloseFile( setup_file);

###### Exit;

##### end;

##### {-- X-R.}

##### readln( setup_file, s1);

##### input_string:= s1;

##### try

###### get_real( a);

##### except

###### ShowMessage(

###### 'Expected real number in '+ S1);

###### CloseFile( setup_file);

###### Exit;

##### end;

##### xf:= roc+ a;

##### x_out. Caption:= FormatFloat( '0.000', a);

##### {-- Y_MM.}

##### readln( setup_file, s1);

##### input_string:= s1;

##### try

###### get_real( y_mm);

##### except

###### ShowMessage(

###### 'Expected real number in '+ S1);

###### CloseFile( setup_file);

###### Exit;

##### end;

##### Y_OUT. Caption:= FormatFloat( '0.000', y_mm);

##### {-- ZONE_Y.}

##### readln( setup_file, s1);

##### input_string:= s1;

##### try

###### get_real( zone_y);

##### except

###### ShowMessage(

###### 'Expected real number in '+ S1);

###### CloseFile( setup_file);

###### Exit;

##### end;

##### {-- Mask type.}

##### readln( setup_file, s1);

##### ch:= upcase( s1[ 1]);

##### case ch of

###### 'c': begin

Item_Index:= 0;

readln( setup_file, s1);

input_string:= s1;

try

get_integer( zones);

get_integer( zone);

except

ShowMessage(

'Expected two integers in '+ S1);

CloseFile( setup_file);

Exit;

end;

mask_height:= exp( 0.25* ln( 0.25* wavelength* sqr( roc)* roc));

zone_n. Caption:= IntToStr( zone);

zone_y:= 0.5* zone_space*( sqrt( zone)+ sqrt( zone- 1));

###### end;

###### 'n': Item_Index:= 1;

###### 'p': Item_Index:= 2;

###### 'r': begin

Item_Index:= 3;

readln( setup_file, s1);

input_string:= s1;

try

get_real( mask_height);

get_real( a);

get_real( b);

except

ShowMessage(

'Expected three real numbers in '+ S1);

CloseFile( setup_file);

Exit;

end;

outer. Caption:= FormatFloat( '0.00', a);

inner. Caption:= FormatFloat( '0.00', b);

zone_y:= 0.5*( a+ b);

###### end;

##### else

###### ShowMessage(

###### 'Expected mask type in '+ S1);

###### CloseFile( setup_file);

###### Exit;

##### end;

##### get_data:= false;

##### mask_typeClick( open_setup);

##### get_data:= true;

##### CloseFile( setup_file);

##### rms_out. Caption:= FormatFloat( '0.0', rms);

#### end;

### end;

## end; {-- open_setupClick}

## procedure TForm1. outerClick( Sender: TObject);

### var

#### a:	extended;

## begin

### a:= zone_y+ 0.5* zone_space;

### input_string:= FormatFloat( '0.00', a);

### if InputQuery( 'Outer zone radius', 'mm <= '+
FormatFloat( '0.0', 0.5* diam), input_string) then begin

#### get_real( a);

#### if a> 0.5* diam then a:= 0.5* diam;

#### zone_space:= a- zone_y+ 0.5* zone_space;

#### zone_y:= a- 0.5* zone_space;

#### outer. Caption:= FormatFloat( '0.00', a);

#### set_zone_data;

### end;

## end; {-- outerClick}

## procedure TForm1. ronchi_linesClick( Sender: TObject);

## begin

### input_string:= InputBox(

'Screen frequency, - for dark centre', 'lines/mm', FormatFloat( '0.000', lines_mm));

### get_real( lines_mm);

### ronchi_lines. Caption:= FormatFloat( '0.000', lines_mm);

## end; {-- ronchi_linesClick}

## procedure TForm1. R_OUTClick( Sender: TObject);

### var

#### a:	extended;

## begin

### input_string:= FormatFloat( '0', roc);

### if InputQuery('Paraxial radius of curvature', 'mm', input_string) then begin

#### a:= xf- roc; get_real( roc); xf:= roc+ a;

#### r_out. Caption:= FormatFloat( '0.000', roc);

#### set_zone_data;

#### rms_out. Caption:= FormatFloat( '0.0', rms);

### end;

## end; {-- r_outClick}

## procedure TForm1. save_imageClick( Sender: TObject);

## begin

### with save_file do begin

#### Title:= 'Save image to .bmp file.';

#### DefaultExt:= GraphicExtension( TBitMap);

#### Filter:= GraphicFilter( TBitMap);

#### if Execute then if Views. ActivePage= m_page then view. Picture. SaveToFile( FileName) else I_view. Picture. SaveToFile( FileName);

### end;

## end; {-- save_imageClick}

## procedure TForm1. save_setupClick( Sender: TObject);

### {-- Save setup file.}

### var

#### izn:	tz_names;

## begin

### with save_file do begin

#### Title:= 'Save setup file.';

#### Filter:=

#### 'Text files (*.set, *.txt)|*.set;*.txt|All files (*.*)|*.*';

#### if Execute then begin

##### AssignFile( setup_file, FileName);

##### rewrite( setup_file);

##### input_string:= InputBox( 'Comment text for first line', '', '');

##### writeln( setup_file, input_string);

##### writeln( setup_file, FormatFloat( '0.0', diam)+

##### #9'D, mm');

##### writeln( setup_file, FormatFloat( '0.000', roc)+

##### #9'R, mm');

##### writeln( setup_file, FormatFloat( '0.000', deformation)+

##### #9'b');

##### writeln( setup_file,

##### 'cos(nA)'#9'sin(nA)'#9'Apn Zernike coefficients, nm');

##### for izn:= a11 to a44 do writeln( setup_file,

##### FormatFloat( '0.0', 1.0e6* re( z_coeff[ izn]))+ #9+ FormatFloat( '0.0', 1.0e6* im( z_coeff[ izn]))+ #9+ z_strings[ izn]);

##### writeln( setup_file, IntToStr( n_points)+

##### #9'NxN FFT');

##### writeln( setup_file, FormatFloat( '0.0', t_width)+

##### #9'pupil width, mm');

##### writeln( setup_file, FormatFloat( '0.000', source_width)+

##### #9'source width, mm');

##### case test_type. ItemIndex of

###### 0: writeln( setup_file,

###### 'ke'#9'test type');

###### 1: begin

writeln( setup_file,

'ronchi'#9'test type');

writeln( setup_file, FormatFloat( '0.00', lines_mm)+

##### #9'lines/mm (- means black center)');

###### end;

###### 2: begin

writeln( setup_file,

'slit/wire'#9'test type');

writeln( setup_file, FormatFloat( '0.00', slit_width)+

##### #9'Width, mm (- means wire');

###### end;

##### end;

##### case source_type. ItemIndex of

###### 0: writeln( setup_file,

###### 'fixed'#9'-source');

###### 1: writeln( setup_file,

###### 'moving'#9'-source');

##### end;

##### with colour do case ItemIndex of

###### 0: writeln( setup_file,

###### 'Red'#9'source color, wavelength '+ FormatFloat( '0.0', 1.0e6* wave_table[ ItemIndex])+

###### ' nm');

###### 1: writeln( setup_file,

###### 'Yellow'#9'source color, wavelength '+ FormatFloat( '0.0', 1.0e6* wave_table[ ItemIndex])+

###### ' nm');

###### 2: writeln( setup_file,

###### 'Green'#9'source color, wavelength '+ FormatFloat( '0.0', 1.0e6* wave_table[ ItemIndex])+

###### ' nm');

###### 3: writeln( setup_file,

###### 'Cyan'#9'source color, wavelength '+ FormatFloat( '0.0', 1.0e6* wave_table[ ItemIndex])+

###### ' nm');

###### 4: writeln( setup_file,

###### 'Blue'#9'source color, wavelength '+ FormatFloat( '0.0', 1.0e6* wave_table[ ItemIndex])+

###### ' nm');

###### 5: writeln( setup_file,

###### 'White'#9'source color, wavelength '+ FormatFloat( '0.0', 1.0e6* wave_table[ ItemIndex])+

###### ' nm');

##### end;

##### writeln( setup_file, FormatFloat( '0.000', xf- roc)+

##### #9'tester X-R, mm (distance to/from RoC, - inside)');

##### writeln( setup_file, FormatFloat( '0.000', y_mm)+

##### #9'tester Y, mm (cross-axis)');

##### writeln( setup_file, FormatFloat( '0.000', zone_y)+

##### #9'zone radius, mm');

##### case mask_type. ItemIndex of

###### 0: begin

writeln( setup_file,

'couder'#9'mask');

writeln( setup_file, IntToStr( zones)+ #9+ IntToStr( zone)+

#9'# zones, at zone #');

###### end;

###### 1: writeln( setup_file,

'none'#9'mask');

###### 2: writeln( setup_file,

'pinstick'#9'mask');

###### 3: begin

writeln( setup_file,

'ritchey'#9'mask');

writeln( setup_file, FormatFloat( '0.0', mask_height)+#9+ outer. Caption+ #9+ inner. Caption+

#9'hole half-height, outer radius, inner radius, all mm')

###### end;

##### end;

##### CloseFile( setup_file);

#### end;

### end;

## end; {-- save_setupClick}

## procedure TForm1. set_zone_data;

### {-- Compute and display Foucault, LWT, and caustic positions.}

### type

#### n_vector= array[ 0.. 1] of extended;

### var

#### a:	 extended;

#### m:	 n_vector; {-- Point on mirror surface.}

#### n:	 n_vector;

#### p:	 extended;

#### r:	 extended; {-- Local radius of curvature.}

#### r_ds:	 extended;

#### ray:	 n_vector;

#### ref:	 extended;

#### x:	 extended;

#### xp:	 extended;

#### y:	 extended;

## begin

### y:= zone_y;

### zone_r. Caption:= FormatFloat( '0.00', y);

### p:= deformation+ 1.0;

### a:= sqr( y);

### x:= a/( roc+ sqrt( sqr( roc)- p* a));

### m[ 0]:= x; m[ 1]:= y;

### xp:= y/( roc- p* x);

### a:= sqrt( 1.0+ sqr( xp));

### n[ 0]:= 1.0/ a; n[ 1]:= -xp/ a;

### r:= IntPower( a, 3)*( roc- p* x)/( 1.0+ p* sqr( xp));

### ref:= roc;

### case source of

#### moving: begin

##### ray[ 0]:= n[ 0]; ray[ 1]:= n[ 1];

##### r_ds:= 1.0/ r;

#### end;

#### fixed: begin

##### r_ds:= 1.0/ sqrt( sqr( roc- x)+ sqr( y));

##### a:= n[ 1]*( roc- x)* r_ds+ n[ 0]* y* r_ds;

##### ray[ 0]:= sqrt( sqr( n[ 0])+ sqr( a)*( sqr( n[ 1]- 1.0)))- a* n[ 1]; ray[ 1]:=( a+ n[ 1]* ray[ 0])/ n[ 0];

#### end;

#### star: begin

##### r_ds:= 0.0;

##### ray[ 0]:= sqrt( sqr( n[ 0])+ sqr( n[ 1])*( sqr( n[ 1])- 1.0))- sqr( n[ 1]); ray[ 1]:= n[ 1]*( ray[ 0]+ 1.0)/ n[ 0];

##### ref:= 0.5* roc;

#### end;

### end;

### f_out. Caption:= FormatFloat( '0.000', m[ 0]- m[ 1]* ray[ 0]/ ray[ 1]- ref);

### l_out. Caption:= FormatFloat( '0.000', m[ 1]+ ray[ 1]*( xf- m[ 0])/ ray[ 0]);

### a:= 1.0/( 2.0/ r/( n[ 0]*ray[ 0]+ n[ 1]*ray[ 1])- r_ds);

### cx_out. Caption:= FormatFloat( '0.000', m[ 0]+ a* ray[ 0]- ref);

### cy_out. Caption:= FormatFloat( '0.000', m[ 1]+ a* ray[ 1]);

## end; {-- set_zone_data}

## procedure TForm1. slit_wClick( Sender: TObject);

## begin

### input_string:= InputBox( 'Slit width, - for wire', 'mm', FormatFloat( '0.000', slit_width));

### get_real( slit_width);

### slit_w. Caption:= FormatFloat( '0.000', slit_width);

## end; {-- slit_wClick}

## procedure TForm1. source_typeClick( Sender: TObject);

## begin

### case source_type. ItemIndex of

#### 0: begin

##### source:= fixed;

##### xf:= roc;

#### end;

#### 1: begin

##### source:= moving;

##### xf:= roc;

#### end;

#### 2: begin

##### source:= star;

##### item_index:= 5;

##### colourClick( source_type);

##### source_width:= 0.0;

##### sw_out. Caption:= FormatFloat( '0.0', 0.0);

##### xf:= 0.5* roc;

#### end;

### end;

### x_out. Caption:= FormatFloat( '0.0', 0.0);

### if source<> star then x_label. Caption:= 'X - R, mm'  else x_label. Caption:= 'X - F, mm';

### set_zone_data;

### compute. SetFocus;

## end; {-- source_typeClick}

## procedure TForm1. sw_outClick( Sender: TObject);

## begin

### input_string:= FormatFloat( '0.000', source_width);

### if InputQuery( 'Source width >= 0', 'mm', input_string) then begin

#### get_real( source_width);

#### sw_out. Caption:= FormatFloat( '0.000', source_width);

### end;

## end; {-- sw_outClick}

## procedure TForm1. test_typeClick( Sender: TObject);

## begin

### with test_type do begin

#### if sender= open_setup then ItemIndex:= item_index;

#### case ItemIndex of

##### 0: begin

###### test:= ke;

###### RONCHI_LABEL. Visible:= False; ronchi_lines. Visible:= False;

###### SLIT_LABEL. Visible:= False; slit_w. Visible:= False;

##### end;

##### 1: begin

###### test:= ronchi;

###### if get_data then ronchi_linesClick( test_type);

###### RONCHI_LABEL. Visible:= True; ronchi_lines. Visible:= True;

###### SLIT_LABEL. Visible:= False; slit_w. Visible:= False;

##### end;

##### 2: begin

###### test:= slit;

###### if get_data then slit_wClick( test_type);

###### RONCHI_LABEL. Visible:= False; ronchi_lines. Visible:= False;

###### SLIT_LABEL. Visible:= True; slit_w. Visible:= True;

##### end;

#### end;

### end;

### compute. SetFocus;

## end; {-- test_typeClick}

## procedure TForm1. toggleClick( Sender: TObject);

### type

#### normal=	array[ 1.. 10000] of extended;

#### P_normal=	^normal;

#### temp_array=	array[ 1.. 500] of extended;

#### P_temp=	^temp_array;

### var

#### bi_at_y:	vect100;

#### bm:	integer;

#### bn:	integer;

#### dy:	extended;

#### i:	integer;

#### j:	integer;

#### k:	integer;

#### l:	integer;

#### left:	integer;

#### low:	integer;

#### mat:	P_normal;

#### n_zones:	integer;

#### size:	integer;

#### x:	extended;

#### xi:	P_temp;

#### y:	extended;

#### zone:	P_temp;

## begin

### if use_data then begin

#### {-- Use conic data.}

#### use_data:= false;

#### xf:= xf- roc; roc:= roc_save;

#### deformation:= deform_save;

#### r_out. Caption:= FormatFloat( '0.000', roc);

#### xf:= xf+ roc;

#### diam:= diam_save;

#### d_out. Caption:= FormatFloat( '0.0', diam);

#### b_out. Caption:= FormatFloat( '0.000', deformation);

#### toggle. Caption:= 'Use mirror data';

### end else if data_read then begin

#### {-- Use mirror data.}

#### use_data:= true;

#### roc_save:= roc; xf:= xf- roc; roc:= roc_actual;

#### deform_save:= deformation; deformation:= deform_actual;

#### r_out. Caption:= FormatFloat( '0.000', roc);

#### xf:= xf+ roc;

#### diam_save:= diam; diam:= diam_actual;

#### d_out. Caption:= FormatFloat( '0.0', diam);

#### b_out. Caption:= FormatFloat( '0.000', deformation);

#### toggle. Caption:= 'Use conic data';

### end else with open_file do begin

#### Title:= 'Open surface file.';

#### Filter:= 'Text files (*.txt, *.srf)|*.txt;*.srf';

#### if Execute then begin

##### {-- Read data from Sixtests' surface file, or file with similar structure.}

##### AssignFile( mirror, open_file. FileName);

##### reset( mirror);

##### readln( mirror); {-- Comment}

##### readln( mirror); {-- A}

##### {-- Get b.}

##### readln( mirror, input_string);

##### get_real( deformation);

##### deform_save:= deformation;

##### deform_actual:= deformation;

##### b_out. Caption:= FormatFloat( '0.000', deformation);

##### {-- Get R.}

##### readln( mirror, input_string);

##### get_real( roc_actual);

##### roc_save:= roc_actual;

##### xf:= xf- roc; roc:= roc_actual; xf:= xf+ roc;

##### r_out. Caption:= FormatFloat( '0.000', roc);

##### readln( mirror);

##### {-- Get surface data.}

##### n_zones:= 0;

##### new( zone); new( xi);

##### while not eof( mirror) do begin

###### inc( n_zones);

###### readln( mirror, input_string);

###### get_real( zone[ n_zones]); get_real( xi[ n_zones]);

##### end;

##### diam_actual:= 2.0* zone[ n_zones];

##### diam_save:= diam_actual;

##### diam:= diam_actual;

##### {-- Set up spline fit.}

##### order:= 4;

##### n_knots:= order- 1;

##### for i:= 1 to n_knots do knots[ i]:= 0.0;

##### y:= 0.0; dy:= 5.0;

##### repeat

###### y:= y+ dy;

###### inc( n_knots);

###### knots[ n_knots]:= y;

##### until y>= 0.5* diam;

##### y:= zone[ n_zones]+ 1.0;

##### for i:= 0 to order- 1 do knots[ n_knots+ i]:= y;

##### n_knots:= n_knots+ order- 1;

##### new( mat);

##### size:= n_knots- order;

##### for i:= 1 to size do coef[ i]:= 0.0;

##### for i:= 1 to sqr( size) do mat^[ i]:= 0.0;

##### for k:= 1 to n_zones do begin

###### y:= zone[ k];

###### x:= xi[ k];

###### interv( knots, n_knots, y, left, i);

###### bsplvb( knots, order, 1, y, left, bi_at_y);

###### low:= order+ 1- left;

###### if low< 1 then low:= 1;

###### for i:= low to order do begin

bm:= left- order+ i;

coef[ bm]:= coef[ bm]+ x* y* bi_at_y[ i];

for j:= low to order do begin

bn:= left- order+ j;

l:= size*( bm- 1)+ bn;

mat^[ l]:= mat^[ l]+ y* bi_at_y[ i]* bi_at_y[ j];

end;

###### end;

##### end;

##### dispose( xi); dispose( zone);

##### matrix_size:= size;

##### chol( mat^, i);

##### if i= 0 then solve( mat^, coef) else begin

###### ShowMessage( 'Spline fit failed');

###### exit;

##### end;

##### dispose( mat);

##### data_read:= true; use_data:= true;

##### d_out. Caption:= FormatFloat( '0.0', diam);

##### toggle. Caption:= 'Use conic data';

#### end;

### end;

### compute. SetFocus;

## end; {-- toggleClick}

## procedure TForm1. w_outClick( Sender: TObject);

## begin

### input_string:= FormatFloat( '0', t_width);

### if InputQuery( 'Width', 'mm', input_string) then begin

#### get_real( t_width);

#### w_out. Caption:= FormatFloat( '0', t_width);

#### if mask= pinstick then set_pin_space;

### end;

## end; {-- w_outClick}

## procedure TForm1. x_outClick ( Sender: TObject);

## begin

### if source = star then begin

#### input_string:= FormatFloat( '0.000', xf- 0.5* roc);

#### if InputQuery( 'Tester distance from focus (+outside)', 'mm', input_string) then begin

##### get_real( xf);

##### x_out. Caption:= FormatFloat( '0.000', xf);

##### xf:= xf+ 0.5* roc;

#### end;

### end else begin

#### input_string:= FormatFloat( '0.000', xf- roc);

#### if InputQuery( 'Tester distance from RofC (+beyond)', 'mm', input_string) then begin

##### get_real( xf);

##### x_out. Caption:= FormatFloat( '0.000', xf);

##### xf:= xf+ roc;

#### end;

### end;

### set_zone_data;

### compute. SetFocus;

## end; {-- x_outClick }

## procedure TForm1. y_outClick ( Sender: TObject);

## begin

### input_string:= FormatFloat( '0.000', y_mm);

### if InputQuery( 'Tester Y', 'mm', input_string) then begin

#### get_real( y_mm);

#### Y_OUT. Caption:= FormatFloat( '0.000', y_mm);

### end;

### compute. SetFocus;

## end; {-- y_outClick }

## procedure TForm1. zc_outEnter( Sender: TObject);

### {-- On entry to Zernike list box, enable button signalling exit.}

## begin

### zc_out.Color:= clYellow;

### store. Visible:= true;

## end; {-- zc_outEnter}

## procedure TForm1. zc_outExit( Sender: TObject);

### {-- Save Zernike coefficient changes in memory.}

### var

#### a:	extended;

#### b:	extended;

#### izn:	tz_names;

#### s1:	string;

#### s2:	string;

## begin

### store. Visible:= false;

### with zc_out do begin

#### for izn:= a11 to a44 do begin

##### s1:= Lines[ ord( izn)];

##### input_string:= s1;

##### try

###### get_real( a);

###### get_real( b);

##### except

###### ShowMessage(

###### 'This line has a problem:  '+ s1);

###### Color:= clWhite;

###### exit;

##### end;

##### z_coeff[ izn]:= c_scale( 1.0E-6, cmplx( a, b));

##### s1:= zc_str( a, 6);

##### s2:= zc_str( b, 7);

##### Lines[ ord( izn)]:= s1+ s2;

#### end;

#### Color:= clWhite;

### end;

### rms_out. Caption:= FormatFloat( '0.0', rms);

### compute. SetFocus;

## end; {-- zc_outExit}

## procedure TForm1. zone_nClick( Sender: TObject);

### var

#### i:	integer;

## begin

### input_string:= IntToStr( zone);

### if InputQuery( 'Show zone parameters', 'Zone number <= '+ IntToStr( zones), input_string) then begin

#### get_integer( i);

#### if i<= zones then begin

##### zone:= i;

##### zone_n. Caption:= IntToStr( zone);

##### zone_y:= 0.5* zone_space*( sqrt( zone)+ sqrt( zone- 1));

##### set_zone_data;

#### end;

### end;

## end; {-- zone_nClick}

## procedure TForm1. zone_rClick( Sender: TObject);

## begin

### input_string:= FormatFloat( '0.0', 0.2* diam);

### if InputQuery(

'Test parameters for zone radius', 'y <= '+FormatFloat( '0.0', 0.5* diam)+ ' mm',

### input_string) then begin

#### get_real( zone_y);

#### set_zone_data;

### end;

## end; {-- zone_rClick}

# end.